MODBUS Connection with Node Js

MODBUS Simulator tool  : Modbus Slave

Connection used  : TCP / IP
 
Simulator Setup Steps :

- Open the Slave Simulator

- Create a New Slave by select New button

- Click the Setup and select the slave definition

- Edit the configuration like below

- Select the function and choose the customized function based on your call method in your code and click ok.

- Click on the Connection menu and click connect

- Click Ok and In the Connection setup screen choose the connection (Eg.Modbus Tcp/Ip).

- Click Ok and connection was done.

Read / Write Methods to Simulator : 

readHoldingRegisters

Slave Functions in slave definition  : Holding Registers

Make a call by using the readHoldingRegisters method.

Params : slaveId , startRegister,numRegisters

O/P : Data from Salve table ( Read values: [ 0, 120, 40, 156, 0 ] )

WriteRegisters

Slave Functions in slave definition  : Holding Registers

Make a call by using the WriteRegisters method.

Params : slaveId , registerAddress , valuetoWrite

O/P : Data added in the Salve table .

WriteCoils

Slave Functions in slave definition  : Coil Status

Make a call by using the WriteCoils method.

Params : slaveId , registerAddress , true/false

O/P : Data added in the Salve table .

readCoils

Slave Functions in slave definition  : Coil Status

Make a call by using the readCoils method.

Params :  slaveId ,startRegister ,numRegisters

O/P : Data from Salve table ( data: [

false, true,

true,  true,

false, false,

false, false

],

)

readInputRegisters

Slave Functions in slave definition  : Input Registers

Make a call by using the readInputRegisters method.

Params : slaveId ,startRegister ,numRegisters

O/P : Data from Salve table ( Read values: [ 0, 0, 0, 1 ] )

readDiscreteInputs

Slave Functions in slave definition  : Input status

Make a call by using the readDiscreteInputs method.

Params : slaveId , startRegister, numRegisters

O/P : Data from Salve table ( data: [

false, true,

true,  true,

false, false,

false, false

],

)